Crysteel Loboy Hoist Model C-8 (Charlie)

( NTEA Performance Class C

( NTEA Type I

( Standard double acting cylinder

( Single stage cylinder

( Cylinder bore 5”

( Cylinder stroke 15-1/4”

( 2” cylinder shaft diameter

( Cylinder shaft is chromed SW85 steel, 85,000 PSI yield strength

( Maximum operating pressure 3250 PSI

*Pressure relief not required on the down side, but highly recommended to limit pressure to 500-1000 PSI pressure.

( Base port size for raise 9/16-18 ORB, rod port lower 9/16-18 ORB

( Cylinder displacement up 301.888 cubic inches, cylinder displacement down 253.586 cubic inches

( Mounting height 7.27”

( Rear hinge fabricated with 3-1/2” x 2-1/2” x 1/4” x 36-1/2” structural angle

( Hinge pins 1-5/16” x 4-3/8” C1045 steel shaft with grease zerks   

( Body prop to support empty body weight

( Hoist weight approximately 365 pounds

( Hydraulics: 4 GPM direct mount gear pump, open center valve mounted to a 15-quart capacity reservoir, self-centering locking sealed cable control with pedestal stand

( WARRANTY

Dump body, hoist, and air cylinder to be covered for a period of 5 years.  Warranty begins when the final customer puts our product into service.  This warranty will cover the dump body, hoist, and air cylinder for defective material and/or workmanship at a rate of 100% for the first (3) years and at a rate of 50% for years 4 and 5.
