Crysteel Loboy Hoist Model E-15 (Ernie)

( NTEA Performance Class E

( NTEA Type I

( Standard double acting cylinder

( Single stage cylinder

( Cylinder bore 6”

( Cylinder stroke 20.688

( 2” cylinder shaft diameter

( Cylinder shaft is chromed SW85 steel, 85,000 PSI yield strength

( Maximum operating pressure 3250 PSI

*Pressure relief not required on the down side but highly recommended to limit pressure to 500-1000 PSI pressure

( Internal bypass to protect cylinder from damage

( Port sizes, SAE-8 (3/4 – 16 ORBM)

( Cylinder displacement up 585 cubic inches, cylinder displacement down 520 cubic inches

( Mounting height 7.83”

( Rear hinge fabricated with 5” x 3” x 3/8” x 36-1/2” structural angle

( Hinge pins 1-3/4” x 5-13/16” C1045 steel shaft with grease zerks   

( Body prop to support empty body weight

( Hoist weight approximately 430 pounds

( Hydraulics: 6 GPM direct mount gear pump, open center valve mounted to a 6-gallon capacity reservoir, self-centering locking sealed cable control with pedestal stand

( WARRANTY

Dump body, hoist, and air cylinder to be covered for a period of 5 years.  Warranty begins when the final customer puts our product into service.  This warranty will cover the dump body, hoist, and air cylinder for defective material and/or workmanship at a rate of 100% for the first (3) years and at a rate of 50% for years 4 and 5.
