Crysteel Loboy LB400 Hoist

( NTEA Performance Class B

( NTEA Type I

( Standard double acting cylinder

( Single stage cylinder

( Cylinder bore 4”

( Cylinder stroke 15-1/4”

( 1.5” cylinder shaft diameter

( Cylinder shaft is chromed SW85 steel, 85,000 PSI yield strength

( Maximum operating pressure 3250 PSI

( Base port size for raise 9/16-18 ORB (.078 Orifice), rod port lower 9/16-18 ORB

( Cylinder displacement up 193.208 cubic inches, cylinder displacement down 166.038 cubic inches

( 45 or 50 degree dumping angle available

( 9” mounting height

( Full-length subframe 

( Subframe has 3-3/4” tall, fabricated “C” channel frame rails  

( Subframe rails fabricated of 10ga A1011 steel 50,000 PSI yield, 65,000 PSI tensile strength

( Rear hinge fabricated with 3.5” x 2.5” x 1/4” x 36-1/2” structural angle

( Hinge pins 1-5/16” x 4-3/8” C1045 steel shaft with grease zerks   

( Body prop to support empty body weight

( Hoist weight approximately 425 pounds

( Hydraulics: 4 GPM direct mount gear pump, open center valve mounted to a 15-quart capacity reservoir, self-centering locking sealed cable control with pedestal stand

( WARRANTY

Dump body, hoist, and air cylinder to be covered for a period of 5 years.  Warranty begins when the final customer puts our product into service.  This warranty will cover the dump body, hoist, and air cylinder for defective material and/or workmanship at a rate of 100% for the first (3) years and at a rate of 50% for years 4 and 5.
