Crysteel Loboy LB520 Hoist

( NTEA Performance Class D

( NTEA Type I

( Standard double acting cylinder

( Single stage cylinder

( Cylinder bore 5”

( Cylinder stroke 21-5/8”

( 2” cylinder shaft diameter

( Cylinder shaft is chromed SW85 steel, 85,000 PSI yield strength

( Maximum operating pressure 3250 PSI

( Base port size for raise 7/8-14 ORB, rod port lower ¾-16 ORB

( Cylinder displacement up 424.607 cubic inches, cylinder displacement down 356.670 cubic inches

( 45 or 50 degree dumping angle available

( 12” mounting height

( Full-length subframe 

( Subframe has 5” tall, fabricated “C” channel frame rails  

( Subframe rails fabricated of ¼” A1011 steel 50,000 PSI yield, 65,000 PSI tensile strength

( Rear hinge fabricated with 3” x 5” x 3/8” x 36-1/2” structural angle

( Hinge pins 1-3/4” x 5-7/8” C1045 steel shaft with grease zerks   

( Body prop to support empty body weight

( Hoist weight approximately 635 pounds

( Hydraulics: 4 GPM direct mount gear pump, open center valve mounted to a 6-gallon capacity reservoir, self-centering locking sealed cable control with pedestal stand

( WARRANTY

Dump body, hoist, and air cylinder to be covered for a period of 5 years.  Warranty begins when the final customer puts our product into service.  This warranty will cover the dump body, hoist, and air cylinder for defective material and/or workmanship at a rate of 100% for the first (3) years and at a rate of 50% for years 4 and 5.
