Crysteel Loboy Hoist Model ST4000

( NTEA Performance Class H

( NTEA Type I

( Twin cylinder hoist

( Standard double acting cylinders

( Single stage cylinders

( Cylinder bore 5”

( Cylinder stroke 27-1/4”

( 2-3/8” cylinder shaft diameter

( Cylinder shaft is chromed C1050 steel, 100,000 PSI yield strength

( Maximum operating pressure 4000 PSI

( Internal bypass to protect cylinder from damage

( Port sizes, 3/8” NPT

( Cylinder displacement up 1070.108 cubic inches, cylinder displacement down 828.664 cubic inches

( Mounting height 10-1/2”

( Rear hinge fabricated 

( Adjustable rear hinge pivots   

( Dual body props to support empty body weight

( Hoist weight approximately 1119 pounds

( Hydraulics: 10 GPM direct mount gear pump, open center valve mounted to a 8-gallon capacity reservoir, self-centering locking sealed cable control with pedestal stand

( WARRANTY

Dump body, hoist, and air cylinder to be covered for a period of 5 years.  Warranty begins when the final customer puts our product into service.  This warranty will cover the dump body, hoist, and air cylinder for defective material and/or workmanship at a rate of 100% for the first (3) years and at a rate of 50% for years 4 and 5.
