CRYSTEEL STRINGRAY HOIST


Crysteel Stingray 4400 NTEA class H/80


A.
Operating pressure of 3250 PSI

B. 45 degree dumping angle

C. Telescopic twin cylinders


D.
Single acting cylinders (optional double acting)

      E.
Lifting capacity 17.6 – 38.5 tons, dependant on body length, dump angle, and rear body overhang


F.
Two body props to support empty body weight


G.
2 stages and 47” stroke

      HYDRAULICS


A.
Direct mount gear pump is standard


B.
10 gpm at 1000 rpm

C. 8 gallon reservoir

WARRANTY

Dump body, hoist, and air cylinder to be covered for a period of 5 years.  Warranty begins when the final customer puts our product into service.  This warranty will cover the dump body, hoist, and air cylinder for defective material and/or workmanship at a rate of 100% for the first (3) years and at a rate of 50% for years 4 and 5.
