CRYSTEEL TURBO HOIST

      Crysteel T5380 in NTEA class 60


A.
Operating pressure of 2500 PSI


B.
50-degree dumping angle


C.
Single acting


D.
3 stages (5”, 4”, 3”) and 80” stroke


E.
Lifting capacity 23.1 tons


F.
One body prop to support empty body weight

G. 10” interlocking channel longitudinal required

The hoist cylinder shall incorporate the “Auto Bleed” technology, an internal spring assisted device to automatically remove air from the cylinder and hydraulic lines whenever the cylinder is extended and retracted.  The device shall not bleed air directly to atmosphere and shall use hydraulic fluid to push air out of the cylinder and back to tank.

      HYDRAULICS


A.
Direct mount gear pump is standard


B.
10 gpm at 1000 rpm

C. 16-gallon reservoir

WARRANTY

Dump body, hoist, and air cylinder to be covered for a period of 5 years.  Warranty begins when the final customer puts our product into service.  This warranty will cover the dump body, hoist, and air cylinder for defective material and/or workmanship at a rate of 100% for the first (3) years and at a rate of 50% for years 4 and 5.

