CRYSTEEL TURBO HOIST


Crysteel T73129 in NTEA class 120


A.
Operating pressure of 2500 PSI

      B.
50-degree dumping angle


C.
Single acting

C. Lifting capacity 59.3 tons on a 15’ body 

D. Two body props to support empty body weight

E. Front height to be 58” minimum

F. 3 stages (7”, 6”, 5”,) and 129” stroke

G. 12” understructure needed  


The hoist cylinder shall incorporate the “Auto Bleed” technology, an internal spring assisted device to automatically remove air from the cylinder and hydraulic lines whenever the cylinder is extended and retracted.  The device shall not bleed air directly to atmosphere and shall use hydraulic fluid to push air out of the cylinder and back to tank.

      HYDRAULICS


A.
Direct mount gear pump is standard


B.
25 gpm at 1000 rpm


C.
40-gallon reservoir

WARRANTY

Dump body, hoist, and air cylinder to be covered for a period of 5 years.  Warranty begins when the final customer puts our product into service.  This warranty will cover the dump body, hoist, and air cylinder for defective material and/or workmanship at a rate of 100% for the first (3) years and at a rate of 50% for years 4 and 5.

